#### 2.6.7.2 Estimated Groundwater Through Flow & Velocity Rates

Groundwater through flow can estimated approximately wherever there is a predominant groundwater flow direction and independent estimates of the following parameters –

- Mean hydraulic conductivity,

- Mean saturated thickness, and

- Mean groundwater hydraulic gradient.

Top 10 m to 15 m Depth of Sediments

These parameter values were obtained from the following sources. The hydraulic conductivity was average along the main groundwater flow path by taking a profile through interpolated horizontal hydraulic conductivity in Figure 30, the mean saturated thickness was estimated from hydrological setting discussion in Section 2.4 Geology with Figure 7, Section 2.6.1, and Section 2.6.5, while the mean hydraulic gradient was obtained from an extract from Figure 32 based on corrected November groundwater elevations. The flow from perpendicular to the Cowan Block’s principal boundaries (Collins Creek and the Northern Boundary Drain) is approximately 800 m.

The basic equation for the calculation of through flow is as follows –

Q =	(K b) i W

=	(9.14 m/d x 10 m) x 0.0083 x 800

=	607 m3/d (or 7 L/s)

Where:

K =	Mean horizontal hydraulic conductivity

=	9.14 m/d

b =	Mean saturated thickness

=	10 m

i =	Mean groundwater hydraulic gradient

=	0.0083 m/m

W =	Groundwater flow front

=	800 m

Basal Gravels (elevations from -5 m to -20 m MSL)

Q =	(K b) i W

=	(56 m/d x 15 m) x 0.002 x 800

=	1,344 m3/d (or 15.5 L/s)

Where:

K =	Mean horizontal hydraulic conductivity

=	56 m/d

b =	Mean saturated thickness

=	15 m

i =	Mean groundwater hydraulic gradient

=	0.002 m/m

W =	Groundwater flow front

=	800 m

Note: The flatter hydraulic gradient is taken from that measured between existing groundwater investigation bores NB-7 (Deep) and WS3200D that are located to the north of the Cowan Block (see Figure 1).

Basis for estimating Basal Gravel Hydraulic Conductivity and Transmissivity

The basal gravels (represented in Layer 2 of the MODFLOW model from elevations -5 m to -20 m MSL) has not be the subject of aquifer testing or slug testing within the Cowan Block. However, an aquifer test using bores numbered NB-7 and NB-7(OBS). The test bore had a total depth of 24.7m and extended to the base of Holocene sediments to the decomposed sandstone basement. The test bore was screened from depths of 18.7 m to 24.7 m BGL (-10.1 m to -16.1 m MSL), which falls into the range of the basal gravel (model Layer 2) in the site description.

The bore log of NB-7 is shown below in Appendix A, and the logged clean sandy gravel most representative of the basal gravel was encountered at a depth of 11 m BGL (-2.4 m MSL). An observation bore labelled NB-7(OBS) was drilled to a similar depth and screened to overlap with NB-7. The observation bore was placed at a radius was 15.2 m.

A 150 minute (2.5 hours) at a discharge rate of 206 m3/d (2.4 L/s). Groundwater level was monitored in the pumped bore and observation bore during the pumping phase and recovery.

Table 1: Summary of NB-7 pumping test interpretation

|  |  |  | Transmissivity (m2/d) |  | Hyd. Conductivity (m/d) |  |  | Storage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bore # | Duration (min) | Rate (m3/d) | Drawdown | Recovery | Drawdown | Recovery | Storativity |  |
| NBH-7 | 150 | 206 | 7.1 – 6.3 | 62.8 | 1.8 – 1.05 | 10.5 | _ |  |
| NBH-7(OBS) | ------------- | --------- | _ | 337 | _ | 56.1 | 2.4 x 10-4 |  |

Note: Saturated thickness is assumed to be coincident with the length of the test bore screen.

Using the screen length as the estimate of saturated thickness is reasonable in the case of a terrestrial, clast imbricated alluvium that is expected to have significant vertical anisotropy. Consequently the observed recovery (residual drawdown) data at a radius of 15.2 m that derived a transmissivity estimate of 337 m2/d and implied a mean hydraulic conductivity of 56 m/d.

Currently the last paragraph of section 2.6.7.2 reads as follows:

“The through flow and velocity estimates indicates relatively slow groundwater transmission rates. Whereas the surface water drainages carried water at rates approximating 100 L/s and higher with mean open water velocities in range of 0.05 to 0.20 metres per second (m/s), the various estimated groundwater transmission rates from east to west across the Cowan Block were estimated to be in the order of 10 L/s and 80 m/year. This is a strong contrast in hydrological capacity, which suggests that the groundwater system beneath the Coates Block plays a significantly lesser role in effective land drainage.”

The hydraulic gradient is taken from the gradient measured in 1990 between bores NB-7 and WS3200D installed along Burke Road into the basal gravels. Using this default approach the addition of the basal gravel layer throughflow results in the combined throughflow to increase as follows –

Table 2: Combined Throughflow Rate Calculation, adding Basal Gravel Layer

| Throughflow Calculation Layer | Throughflow Rate [m3/d (or L/s)] |
| --- | --- |
| Top 10 m to 15 m Depth of Sediments Throughflow | 607 m3/d (or 7 L/s) |
| Basal Gravel (model Layer 2) Throughflow | 1,344 m3/d (or 15.5 L/s) |
| Combined Throughflow | 1951 m3/d (or 22.6 L/s) |

The sole substantive alteration to the current summary of throughflow and flow velocity (last paragraph of section 2.6.7.2) is that the order of throughflow should be 20 L/s compared to 10 L/s. The observations as to the difference between surface water and groundwater drainage rates should still hold.

The geographic scope of the combined throughflow calculation is mapped in the revised Figure 32 below. The flow path along which the throughflow calculations are based is marked as the broken line starting at the “Proposed Access Road”, passing through the core of the Cowan Block to the coastal lagoon (groundwater elevations of 12.5 m to 1.82 m AMSL). The flow front width through the Cowan Block was estimated at 800 m across.

Figure 32: Groundwater level contours drawn from groundwater elevations mapped across the Cowan Block

(Revised and reviewed re-issue of Figure 32)

Basis for estimating the thickness of Basal Gravels and depth of Blue Bottom Formation

In relation to WGANZ Request For Information (RFI) number 4.14

“Please provide an indication of the thickness of the basal gravel unit underlying the mineral sands. Please also advise what underlies the gravel unit and at what depth the Blue Bottom Formation may be found beneath the site.”

No drilling investigations beyond a couple of metres below the mineral sand – basal gravel contact were pursued to establish the depth and base of the basal gravels within the Cowan Block. However, limited information was available from the Coffey Partners (1991) reporting. The bore log for NB-7 with a recorded depth range from 0 m to 24 m BGL is reproduced below. The bore log provides indications of the top of the basal gravels being at 11 m BGL (-2.4 m MSL) and extending to 24 m BGL. Bore log WS3200D extended to 30.5 m BGL (-26.5 m MSL) yet did not encounter Blue Bottom basement. Both bores were drilled along Burke Road, approximately 1.2 km north of the Cowan Block Northern Boundary Drain or southern shore of Rusty Lagoon. Figure 1 illustrates the relative depths of NB-7 and WS3200D, including the contact between basal gravels and Blue Bottom Formation.

Figure 1: Burke Road profile (cross-section) showing bores NB-7 and WS3200D extending to depth through mineral sands and basal gravels (from Coffey Partners, 1991)

The saturated thickness of basal gravels at bore NB-7 is about 13 m. The bore log for WS3200D indicates basal gravel saturated thickness is between 15 and 17 m. The groundwater measured in NB-7 and WS3200D during 1990 was measured as having electrical conductivities of approximately 380 mS/cm, indicative of a lack of seawater intrusion and fresh groundwater.

Appendix A – Bore Log for NB-7 (Coffey Partners, 1991)

NB-7 Test Bore on Burke Road (June 1990)