Memo

| To: | Brett Sinclair |
| --- | --- |
| From: | Zeb Etheridge & Jens Rekker |
| CC: |  |
| Date: | 27/07/2023 |
| Subject: | RE WGA Peer Review of TiGa resource consent application – Barrytown Mine |

| WGA memo No. | Request summary | Files in support |
| --- | --- | --- |
| 3. 1 | Please provide a map with contours of the projected cumulative pit floor and confirm the maximum depth of mining below ground level. | Written response |
| 3.1 Response | Palaris provided a PDF contour map of the AMSL/RL contours of the base of the excavation. This shows the maximum contour to be -8 m MSL in an isolated patch. About 70% of the depth contours are greater than -5 m MSL, ranging between -1.4 m and -5 m MSL for the southern and eastern parts of the sand extraction area. A nominal -5 m MSL is used as the base of the sand extraction in the MODFLOW model. The maximum depth of excavation is estimated to be approximately 14 from the peak of the peak of a “hump” in the “Hump & Hollow“ topography. | File: 3-1 Floor of Sand_Roof of Gravel.PDF Additional - File: 4-11 Mining Depth From Topography.PDF |
| 4. 2 | A map identifying the locations of both flow monitoring sites | Written response |
| 4.2 Response | The flow sites are located on Collins Creek. The upstream site is immediately upstream of the SH6 bridge and called Collins Upstream or Colins U/S. The downstream site is located at the farm lane ford across Collins Creek and called Collins Downstream or Collins D/S. | File: 4-2_CollinsCk_ Flowsites.PDF |
| 4. 3 | Flow gauging records and the flow rating curves developed for each of the stream flow monitoring sites | Written response |
| 4.3 Response | Rating curve generation was generated from four gauging instances, but the scatter around the linear best fit was significant to result in an R2 of between 0.79 and 0.66. Rating curve generation was problematic. Ultimately, a rating was taken from two gauging instances (11/05/2022 and 7/02/2023). | Files: 4-3_Rating_US.xlsx and 4-3_Rating_DS.xlsx |
| 4. 4 | Please provide a chart showing the difference in calculated creek flows between the two monitoring sites through the duration of the recorded period. | Written response |
| 4.4 Response | The chart below shows the difference between upstream and downstream creek discharge. A Powerpoint file is attached containing the chart graphic. The period covered is 4 May – 25 August 2023, which provides an indication of the correspondence. | File: 4-4_CreekDischarge Diff_Chart.pptx |
| 4. 5 | Please provide the PB-1 test water level curve for both the abstraction test and the injection test | Written response |
| 4.5 Response | PB-1 was a 125 mm (5 inch) diameter testing bore that would not resemble the proposed injection wells at the Cowan Block, Barrytown. This small bore diameter also hampered the ability to take even manual water depth measurements. The injection of creek water into the screened section did not represent an injection test. Only manual measurements of PB-1 water level could be made with some difficulty due to the slender space between injection hose (and couplings) and the internal diameter of the bore casing, and therefore level measurements were not taken during injection. The injection of creek water was undertaken as a last development stage of the test bore before testing began. We noted that the bore did not at any stage during injection overflow, thus indicating a maximum mounding effect. Final pumping step depths to water and drawdowns were available from the pumped bore PB-1, i.e., one final drawdown per step for four steps. Pumped bore depths to water during the constant rate tests were not taken as the objective was to measure the water levels and drawdowns in the observation bores TB-1, TB-2 and TAC-157. | File: 4-5 Step Drawdown Final Drawdown plot.xlsx |
| 4. 6 | Please provide the groundwater up-coning response curves for these bores and an indication of the extent and magnitude of up-coning achieved during the test | Written response |
| 4.6 Response | See response for 4.5. Injection testing was not recorded beyond measuring the injection rate at the beginning of injection. |  |
| 4. 7 | Please provide a reviewed version of the interpreted piezometric map. The aspect of primary interest is what proportion of the groundwater currently flowing across the planned mine pit area is reporting to either the Northern Boundary Drain or Rusty Lagoon. | Written response |
| 4.7 Response | Figure 32 in the hydrological assessment report was found to be in error. The error was a stray line between contour 7 m and 8 m AMSL that gave the appearance or impression that the hydraulic gradient against the Northern Boundary Drain was steeper than measured and corrected level measurements in fact indicated. The stray line has been removed and the editing reviewed, with a PPT file included showing the edited Figure 32, plus Figure 31 and Figure 35 for easy reference to information presented of relevance to the Northern Boundary Drain. | File: 7_Edited Fig32 gw Contour Map.pptx |
| 4. 8 | Please provide a calculation of groundwater through-flow within the basal gravel bed underlying the site, taking into account the transmissivity values derived from the pumping test | Written response |
| 4.8 Response | It was not our belief that the PB-1 pumping test accessed the hydraulic properties of the basal gravel layer. PB-1 was screened to a depth elevation of -0.95 m MSL, while project top of the basal gravels is -4.2 m MSL, thus 2022 pumping tests did not extend into the basal gravels. Instead, the sole pumping test of the basal gravel at a site approximately 1.2 km to the north of the Northern Boundary Drain at bore NB-7, which had a depth of 24.7 m BGL and was screened from 18.7 m BGL (-10.1 M MSL) to 24.7 m BGL (-16.1 m MSL). The bore log showed the screened section was clearly consistent with the basal gravels described elsewhere (” GRAVEL, sandy, fine to coarse gravel and cobbles”). A 2½ hour, 2.4 L/s pumping test determined a transmissivity of 340 m2/d. The hydraulic gradient of the deep (basal gravel) along Burke Road (bores WS3200D and NB-7) was measured at 0.002 m/m. These values were transferred into the Cowan Block throughflow calculation and the deep throughflow added to the throughflow calculated for the shallower layer. | File: 4.8_Basal_gravel_Layer2_Throughflow_Calc.docx |
| 4. 9 | Please reassess the relative combined groundwater flow estimates compared to the surface water flows, as documented in the final paragraph of Section 2.6.7.2. | Written response |
| 4.9 Response | Instead of groundwater throughflow being estimated at 1/10 of surface water discharge, the throughflow estimate taking a conservatively high estimate of basal gravel throughflow places combined throughflow at about ¼ of surface water discharge. | File: 4.8_Basal_gravel_Layer2_Throughflow_Calc.docx |
| 4. 10 | Please provide a contour map of the interface between the base of the mineral sand deposits into the top of the basal gravel deposits. | Video response |
| 4.10 Response | Palaris provided a PDF contour map of “the base of sand surface is the roof of the gravel surface.” | File: 4-10 Insitu Sand Post Mining.PDF |
| 4. 11 | Please provide a contour map of the thickness of mineral sands left in place between the base of the pit shell (as per Request 1 above) and the top of the basal gravels underlying the pit. | Video response |
| 4.11 Response | Palaris provided a contour map of the “vertical thickness contour map between the floor of sand and topography”. This contour map shows the maximum depth from surface topography would be 14 m. However, the Hump & Hollow topography is a significant influence on the peak difference between pit floor and ground surface. Post-stripping depths are likely to be significantly less and less volatile. | File: 4-11 Mining Depth From Topography.PDF |
| 4. 12 | Please advise whether the sump as described above is also to collect water extruded from the accumulating tailings backfill. | Written response |
| 4.12 Response | Palaris provided the following response: It (the sump) is designed so that water from the tails will be collected with work area run off and ground water ingress at the same sump with all water being pumped from the pit sump back to the Mine Water Facility. This consolidates all water management within the working pit area into one location and reporting back to the Mine Water Facility for re-sure and water quality management. Stephen Miller, General Manager, APAC Metal / Palaris, 18 July 2023. |  |
| 4. 13 | Please provide an estimate of mine discharge water quality based on estimates of relative mass load contributions from the various contaminant sources to the mine water management system. In lieu of a calculated discharge based on contributing mass loads, please provide an indication of the ‘worst case’ contaminant concentrations in the mine discharge water and the reasoning behind the values provided. | Written response |
| 4.13 Response | Should the discharge of tails dissolved metals be encouraged to infiltration from the tailings in the wake of the mining in each mine panel, the solutes may enter and combine with the groundwater within the basal gravels under pre-mining vertical hydraulic gradients. In this case, the more likely site of groundwater discharge from the deeper basal gravels aquifer would be at the coastline, into the Tasman Sea. Section 4.2.2.2 of the assessment document states that “A proportion of the seepage may discharge into the coastal lagoon, but most of the influx to the lagoon comprises inflows from Collins Creek and hence any seepage from backfilled material to the lagoon would be diluted significantly. The estimated groundwater throughflow rate at the site is 10 L/s (see Section 2.6.7.2). The mean flow of Collins Creek is estimated to be ~ 50 L/s (Section 2.5). Assuming that all groundwater throughflow beneath the site discharges to the lagoon (which is unlikely to be the case, a significant proportion is likely to flow underneath the lagoon and discharge at the coast), the rate of dilution with Collins Creek water will be at least fivefold, giving a maximum concentration of 0.04 mg/L. This is below the ANZG screening value of 0.055 mg/L.” The revised throughflow analysis envisages the following throughflow rates - 7 L/s in Layer 1 15.5 L/s in Layer 2, most likely discharging at the coast. The revision would see the superficial groundwater throughflow to the Coastal Lagoon change from 10 L/s to 7 L/s and therefore a reduction of the discharge containing solutes discharging into the lagoon. Hence the dilution would be sevenfold and the maximum concentration would be 0.03 mg/L, still beneath the ANZG screening value of 0.055 mg/L. |  |
| 4. 14 | Please provide an indication of the thickness of the basal gravel unit underlying the mineral sands. Please also advise what underlies the gravel unit and at what depth the Blue Bottom Formation may be found beneath the site. | Video response |
| 4.14 Response | The basal gravel is poorly constrained by available drilling information, especially within the boundaries of the Cowan Block where the top couple of metres of the basal gravels have been the limit of drilling. Drilling along Burke Road to the north of the Cowan Block has constrained the absolute and minimum depths of the basal gravels to some extent. The Blue Bottom Formation was found directly beneath the basal gravel at 24 m BGL (-15.4 m MSL). The top of the basal gravel was found at approximately 11 m BGL (-2.4 m MSL) meaning the measured thickness of basal gravel was 13 m between upper contact and basement contact. | 4.8_Basal_gravel_Layer2_Throughflow_Calc.docx |
| 4. 15 | If the direction of groundwater flow within the basal gravel aquifer between the mine and the coastline is at least locally reversed due to drawdown linked to mining operations, what implications will this have for saline water intrusion risks? | Video response |
| 4.15 Response | The precise tipping points of reversal of groundwater flow is difficult to quantify in relation to Layer 2 and the extent of the Cowan Block. A rough estimate of the Layer 2 pore volume of groundwater is 1,500 x 800 x 15 x 0.20 = 3,600,000 cubic metres at 20% effective porosity. Coffey Partners (1991) found at least two lines of evidence that suggest the groundwater is fresh at the coastline or beyond the coast; a) deep (+30 m BGL) groundwater sampling bores at the coastline returned fresh water samples, and b) electrical resistivity profiling in the late 1980s encountered no indications of saline or brackish groundwater despite the profile transect lines being arranged on and parallel to the coastline and in the foredune zone. Hence, landward movement of the seawater – freshwater interface (transition zone) would be required. Several million cubic metres of groundwater displacement by dewatering, in addition to the 490,000 cubic metres per annum flow from the west in Layer 2 over an extended period of time to give rise to the conditions of landward movement of the seawater – freshwater interface in the groundwater system. An approach of sand extraction area to within 350 m of the coastline in model Year 4.7 was assessed by examining modelled groundwater vectors and levels within Layer 1 and 2 in the vicinity of the active sand extraction zone. The groundwater level with the centre of the active sand extraction pit were determined (L1 GWL = -4.3m, L2 GWL = 0.14 m MSL) (L1 GWL = 3.72 m, L2 GWL = 3.77 m MSL) (L1 drawdown = 8.02 m, L2 drawdown = 3.63 m MSL). Groundwater vectors in both layers were also examined and found to have been diverted into the area of active pit water management. The figure 4-15_GwVectors_L1_L2.pptx shows the swirling of groundwater vectors around, into and past the sand extraction area. Interpretative flow divides were sketched around the model plane around the active sand extraction area. Groundwater vectors show the threshold from cells flowing towards the coastline and cells flow towards the active sand extraction area approximately 150 m from the area or 200 m from the coastline in both model layers. | File: 4-15_GwVectors _L1_L2.pptx |
| 4. 16 | Please present a modified version of Figure 43 clearly showing all the boundary conditions described in the text with appropriate labels. | Video response |
| 4.16 Response | An edited copy of Figure 43 is provided as a Powerpoint file showing the removal of the stray / superfluous line and repositioning of labels.Boundary annotation is provided in colour coded MODFLOW boundary condition classes: Dark Blue = CHD constant head for sea boundary (layers 1 & 2) Yellow = DRN drain boundaries for Mahers Swamp, NBD and upper parts of Collins Creek Green = RIV river boundaries for coastal lagoons lower NBD and lower Collins Creek Dark Red = WEL Simulated infiltration Trenches (SW and Northern) Black = HNF basement Head No Flow boundary Note: The yellow coloured State Highway 6 (SH6) is not a boundary, merely a geographic feature. A PowerPoint file is provided mapping and annotating the TMR | File: 4-16_Fig43_annotation. pptx 4-16 TMR GWV screenshoot.pdf |
| 4. 17 | Please provide the water balance results and the boundary inflow / outflow results for the refined model to support the model documentation. | Written response |
| 4.17 Response | The basic model water balance is as follows – Description; Inflow (m3/d); Outflow (m3/d). Recharge; 142; 0. Constant Head; 40,738; 33,318. River; 13,449; 17,458. TOTAL; 54,329; 54,305. ERROR; 0.043. The breakdown beyond this is somewhat problematic due to the TMR exchanges and balancing with other boundaries. A breakdown of seepage removals of model groundwater during sand extraction operations is summarised below - Boundary; Seep (m3/d). Mahers DRN; 2,428. NBD DRN; 1,101. Collins Ck DRN; 0. Rusty's RIV; 4,184. CollinsCk Confl RIV; 2,415. CollinsCk Mid RIV; 1,517. NBD Lower RIV; 3,994. The coastal lagoons make a complicated interchange with surrounding constant head boundaries, namely the coastal CHDs. |  |
| 4. 18 | Please provide model set-up documentation for the predictive model that was used to generate the flow results documented in Section 6.1, including detail on the progression of the simulated opencast pit across the site. | Video response |
| 4.18 Response | Sand extraction areas were lain into the 6.2-year transient simulation in GWV as 53 rectangular areas within the mine panels. The first stress period in MODFLOW was steady state and this approximated Scenario 0. A common drain elevation per stress period was set within each rectangle in accordance with the elevation of the pit base (see file “3-1 Floor of Sand_Roof of Gravel.PDF”). The pit advance along each mine panel was from west to east, and progressed northward from mine panel 1. The fluxes within the sand extraction operations period were obtained using boundary reach reports from GWV. Depletions were calculated by subtracting change from the initial steady state stress period. | File: 4-19 Model Schematic.pptx 3-1 Floor of Sand_Roof of Gravel.PDF |
| 4. 19 | Please provide the layout of the parameter zones listed in Table 22 within the refined model mesh area. | Video response |
| 4.19 Response | As outlined in other parts of the report describing groundwater flow model set-up, five zones of hydraulic conductivity in the superficial model Layer 1. Parameter; Optimised Value (m/d). Global Recharge; 36.5 mm/yr. Zone 1 Kh; 2.86. Zone 3 Kh; 5.4. Zone 4 Kh; 11. Zone 5 Kh; 80. Zone 6 Kh; 1.9. Note: Zone 2 is in model Layer 2 Zone 3 relates to the Eastern Gravel Overburden deposit the does not extend into the sand extraction area. Zone 4 is a fringing halo of the Eastern Gravel Overburden that was found in optimisation to be more permeable. Zone 5 a high permeability zone suggested by optimisation. It is distributed around the coastal lagoon margins and section of middle Collins Creek. Zone 6 is correlated with the low permeability mineral sand, which is a fine sand considered to have hydraulic conductivity of 3 m/d or less. Zone 1 and Zone 6 are adjacent in terms of hydraulic conductivity and physical distribution. The two figures below show the progression from hydro stratigraphic modelling outlined in section 2.6.6 of the assessment report. Hydro stratigraphic Modelling of Layer 1 Hydraulic Conductivity Hydraulic Conductivity Zonation in Layer 1 The above figures are in the same position and orientation for reference. | File: 4-19_HydCond_ Plots.pptx 4-19_Hydrostrat_ KDistribution.PDF 4-19_ModelK_Zones.PDF |
| 4. 20 | Please provide a chart showing the flow rates into the simulated opencast pit for Layer 1 and Layer 2 separately. How much water is entering the pit from the basal gravels compared to the mineral sands? What component of this water is direct recharge water to the pit footprint? | Written response |
| 4.20 Response | The sand extraction area at model Year 4.7 examined in relation to groundwater vectors in responses to 4.15 was examined in relation to stratification of the model water fluxes. In layer 1 the drain cells simulating the pumping of water from the excavation requires a flux of 130 L/s to maintain the excavation at the target water elevation of -4.28 m MSL. Only 4.7 L/s of the groundwater extracted at the drain reach originated from Layer 1. Approximately 125 L/s (96.4%) of the groundwater originated in Layer 2 and flowed up into the Layer 1 drain reach from below. |  |
| 4. 21 | Please provide indicative groundwater drawdown maps showing the extent of drawdown when ore extraction is operational in Panel 8 and separately when Panel 9 is operational. Please present the drawdown maps for Layer 1 and Layer 2, together with an overlay of surface water bodies, wetland areas and known springs. | Video response |
| 4.21 Response | Drawdown is more concentrated surrounding the respective active sand extraction areas in the superficial Layer 1 than deeper Layer 2. Panel 8: Layer 1 (above) and Layer 2 (below) Panel 1: Layer 1 (above) and Layer 2 (below) The northern panel induced a deeper and greater drawdown compared to the southern panel. | Files: 4-21 Panel8_layer1.PDF 4-21 Panel8_layer2.PDF 4-21 Panel1_layer1.PDF 4-21 Panel1_layer2.PDF |
| 4. 22 | Please provide model outputs for water losses from adjacent surface water bodies to the mine and compare these to inflow and outflow rates for the surface water bodies in the calibration model. | Written response |
| 4.22 Response | The plot below is the transient or time series of model fluxes drawn from the MODFLOW model simulation that includes the groundwater pumped from the active sand extraction pit and the other eight adjoining boundary groups; Lower Collins Creek, Middle Collins Creek, Lower Northern Boundary Drain (NBD), Upper NBD, Rustys Lagoon, Mahers Wetland and the Coastal (Canoe Creek) Lagoons. Note: the Lower Collins Creek and middle Collins Creek depletion sum to 56 L/s effect on the creek at Day 855. | File: 4-22 FlowDepletion_adj_ waterbodies.PDF |
| 4. 23 | Please provide a vector map of groundwater flow directions in Layer 2 in the area between the mine and the coastline. | Video response |
| 4.23 Response | An approach of sand extraction area to within 350 m of the coastline in model Year 4.7 was assessed by examining modelled groundwater vectors and levels within Layer 1 and 2 in the vicinity of the active sand extraction zone. The attached graphic shows groundwater velocity vectors within Layer 2 in the vicinity of the active sand extraction area. This graphic includes an interpretive trace of the groundwater flow divide. Beneath that graphic is an expanded area showing velocity vectors and the Tasman Sea at Year 4.7. | File: 4-23 Vectormap_Layer2. PDF 4-23 Vectors between Pit & Sea.PDF |
| 4. 24 | The acceptance capacity calculation incorporates a hydraulic conductivity of 25 m/day, which is above the range for the mineral sands. Please clarify the source of this value. Also, please provide the calculation used to determine the radius of influence for the mound generated by the recharge. | Written response |
| 4.24 Response | The 85 m2/d transmissivity result derived for test observation bore TB-2 in the PB-1 pumping test was influential in the specification of a hydraulic conductivity of 25 m/d. The TB-2 observation bore was screened in a layer of SAND, silty, with minor fine gravel. The 8.7 m AMSL water table above the base of the materials found at about 5.2 m AMSL, implying a saturated thickness of 3.5 m. Thus, the adjusted hydraulic conductivity for the silty sand with minor gravel of 24 m/d. It is anticipated that the northern and south-eastern infiltration trenches would be established within the coarser overburden materials, above the mineral sands. |  |
| 4. 25 | Please provide the calculations that generated the estimates for potential flow rates to the recharge wells as listed in Section 6.5.1.2. | Written response |
| 4.25 Response | Section 6.5.1.2 could have made clearer the differences in infiltration wells between the shallower materials (ore sand with minor gravel, model Layer 1) and the deeper materials (sandy basal gravels, model Layer 2). Infiltration wells may be screened to achieve infiltration across both layers or individually screened to access one layer and associated porous material, the feasibility assessments would evaluate efficacy and environmental effects. Depth to water table also contrasts between the shallower and deeper layers accessed by the recharge barrier injection wells. Measurements of static water levels in monitoring bores to the north of the Cowan Block range in over-pressure from deep to shallow bores between 0 m to 0.81, i.e., negative (downward vertical gradients ranging between 0 and -0.038 m/m. The calculated Theis Equation aquifer mounding effect at 0.5 m radius (assuming 1.0 m well diameter) would conform with the following list of parameters (T & S) and pumping rates (Q) - - Ore sand with minor gravel, - T = 30 m2/d, S = 0.25, Q = 1.5 L/s - 7-day mounding = 3.0 m - Gravel in ore sand matrix, - T = 90 m2/d, S = 0.25, Q = 5 L/s - 7-day mounding = 3.8 m - Sandy gravels - T = 300 m2/d, S = 0.25, Q = 15 L/s - 7-day mounding = 3.8 m The injection well system would be the subject of well field feasibility testing and trialling. |  |
| 4. 26 | Figure 56 appears to show the combined injection rates to the four proposed recharge bores located along the northern side of the mine. Please confirm this understanding is correct. | Video response |
| 4.26 Response | Not correct. Figure 56 shows the northern infiltration gallery (i.e., trench) injection rates. |  |
| 4. 27 | Figure 58 appears to show the combined injection rates to the proposed recharge bores located along the southern side of the mine. Please confirm this understanding is correct. | Video response |
| 4.27 Response | Not correct. Figure 58 shows the infiltration gallery (i.e., infiltration trench system along south and west margin) Injection wells would have a support role to the main infiltration trench system in instances of surplus, such as the instances shown in Figure 58 when water produced at the mine pit during the early phase of sand extraction exceeds the 42 L/s capacity of the southern infiltration trench / gallery system. |  |
| 4. 28 | Please provide the MODFLOW model outputs for water losses from adjacent surface water bodies to the mine with the recharge systems in place and operating. Compare these depletion rates for the adjacent water bodies to inflow and outflow rates for the surface water bodies in the calibration model, and similarly to the model with no management measures in place. | Written response Overlaps 4.22. |
| 4. 29 | Please provide a map showing the extent and magnitude of groundwater mounding in Layer 2 that would arise from the simulated recharge bore operation at the northern side of the mine. The mounding should be representative of a date late in the mining period with a high simulated rate of recharge. Please also provide a piezometric map for Layer 2 for the same date. | Video response |
| 4.29 Response | Covered in video conference | See also response for 4.21 |
| 4. 30 | Please provide a map showing the extent and magnitude of groundwater mounding in Layer 2 that would arise from the simulated recharge bore operation at the southern side of the mine. The mounding should be representative of a date early in the mining period with a high simulated rate of recharge. Please also provide a piezometric map for Layer 2 for the same date | Video response |
| 4.30 | Covered in video conference | See also response for 4.21 |
| 4. 31 | Please provide the calculations to support the estimation of a 60 L/s depletion rate from Collins Creek. | Written response |
| 4.31 Response | We anticipate a peak during year 2 in the lower Collins of 44.5 L/s and Mid-Collins of 11.7 L/s, equating to 56 L/s of depletion based on transient simulation using the TMR MODFLOW model. This is the basis for supposing that up to 60 L/s of augmentation might be required. |  |
| 4. 32 | Please provide simple water balance calculations for the mine site |  |
| 4.32 Response | The basic model water balance is complicated by the transient simulation. Mean groundwater exchanges are provided below. The mean operational water balance terms relate to mitigated exchanges (i.e., with infiltration in operation too). The breakdown beyond this is somewhat problematic due to the TMR exchanges and balancing with other boundaries. A breakdown of seepage removals of seepage and seepage change (i.e., depletion) at several during sand extraction operations is summarised below - Boundary; Steady State (m3/d); Operational Mean Seepage (m3/d); Long-Term Depletion (m3/d). Mahers DRN; 2,428; 2,420; 8. NBD DRN; 1,101; 983; 118. Collins Ck DRN; 0; 0; 0. Rusty's RIV; 4,184; 4,047; 137. CollinsCk Confl RIV; 2,415; 1,648; 767. CollinsCk Mid RIV; 1,517; 997; 520. NBD Lower RIV; 3,994; 3,785; 209. |  |

Appendices

List of files provided alongside responses

| 1 | 3-1 Floor of Sand_Roof of Gravel.pdf |
| --- | --- |
| 2 | 4-1_3b_Rating_curve_DS.xlsx |
| 3 | 4-1_Item3a_Rating_curve_US.xlsx |
| 4 | 4-2_CollinsCk_Flowsites.pdf |
| 5 | 4-3_Rating_curve_DS.xlsx |
| 6 | 4-3_Rating_curve_US.xlsx |
| 7 | 4-4_CreekDischargeDiff_Chart.pptx |
| 8 | 4-5 Step Drawdown Final Drawdown plot.xlsx |
| 9 | 4-7_BasicWater Bal.xlsx |
| 10 | 4-7_Edited Fig32 gw Contour Map.pptx |
| 11 | 4-8 Burke Road basement contact.xlsx |
| 12 | 4.8_Basal_gravel_Layer2_Throughflow_Calc.docx |
| 13 | 4-10 Insitu Sand Post Mining.pdf |
| 14 | 4-11 Mining Depth From Topography.pdf |
| 15 | 4-15_GwVectors_L1_L2..pptx |
| 16 | 4-16 TMR GWV screenshoot.pdf |
| 17 | 4-16_Fig43_annotation.pptx |
| 18 | 4-19 Model Schematic.pptx |
| 19 | 4-19_HydCond_plots.pptx |
| 20 | 4-19_Hydrostrat_KDistribution.pdf |
| 21 | 4-19_ModelK_Zones.pdf |
| 22 | 4-20_Schematic.pptx |
| 23 | 4-21 Panel1_layer1.pdf |
| 24 | 4-21 Panel1_layer2.pdf |
| 25 | 4-21 Panel8_layer1.pdf |
| 26 | 4-21 Panel8_layer2.pdf |
| 27 | 4-22 FlowDepletion_adj_waterbodies.pdf |
| 28 | 4-23 Vectormap_Layer2.pdf |
| 29 | 4-23 Vectors between Pit & Sea.pdf |